Radio/Communications (Ground-to-Air, microwave, satellite, telephone, teletype and communications)

FGC-52 Teletypewriter Set

Notes: Off-line tape preparation, on-line transmission

GRC-206 Communications Central

Notes: Self-contained radio sets with UHF-AM, VHF-AM, VHF-FM, and HF-SSB radios. On/off road tactical communications central replaces MRC-107/8.

GRC-206(V3): HF/VHF/UHF communications facility

Range: 30 km, 35 km, or 150 km, depending on antenna used.

Frequency: HF single sideband 2-29.9999MHz; VHF/FM 30-75.975MHz; VHF/AM 116-149.975MHz; UHF 225-399.975MHz
Power Output: HF 150 watts; VHF/FM 2 watts or 10 watts; UHF 2 Watts or 10 Watts

Channels: single channel

Notes: Mounted on an M-998 HMMWV; Remotable up to 2 km. Includes encryption devices

GSQ-120 Microwave

Notes: for remoting radar

Range: 1 kilometer

Power Output: 1 watt

GTA-6 Telephone Central Office Group, Manual

Notes: Manual telephone switching facility

KWM-2A Single Sideband Radio Set

Notes: HF Frequency voice command and control radio (Circa 1959)

MRC-107/108 Communications Central

Notes: Self-contained radio sets with UHF-AM, VHF-AM, VHF-FM, and HF-SSB radios

PRC-??

R-390 Radio Receiver

Notes: http://www.fernblatt.net/390.html

STU-III Secure Telephone Unit

Tape Recorder 24 Hour voice

TRC-24

TRC-32 Air/Ground Communications Sets
Notes: UHF Radio

TRC-36 Point-to-point Radio Relay Set
Notes: Broadband, VHF frequency

TRC-61 Point-to-point Radio Relay Set
Notes: Broadband, VHF frequency

TRC-87 UHF Communications Central

TRC-97 Tropo Scatter Radio Relay
Frequency: 4.4 to 5 Gigahertz

Range: 40 miles, fixed straight line of site; 96 miles Tropospheric

Power Output: 1 watt (using a traveling wave tube amplifier), to 1 kilowatt (using a tunable klystron amplifier)

Antenna: MRT-2 parabolic Tropospheric antenna

Channels: 24 multiplex; 16 teletype

Notes: Broadband, UHF radio set, analog radio

TRC-115 Point-to-point Radio Communication Set

Notes: HF frequency voice and teletype radio

TRC-170 Tropospheric Backscatter Radio

Notes: Point-to-point communications

Range: from 50 to 200 miles

60 channels

TRC-199 Tactical Repeater

Notes: Self-contained/stand-alone unit

TSC-53 Communications System

Notes: HF voice, teletype, and VHF/UHF air-to-ground communications facility.

TSC-60(V) HF Side Band Radio Communications System

Power Output: Two 1 KW transmitters
Channels: four simultaneous voice frequency channels, combination of 4, 8, or 16 teletype channels, and 8 channels of speech-plus teletype.

Notes: Used for ground-to-ground or ground-to-air communications; Two receivers; voice, CW, teletype or high speed data, multiplexed teletype, and speech-plus teletype signal capability;

TSC-62 Technical Control Van

TSC-94A(V)2 Satellite Communications System

TSC-100A(V)2 Satellite Communications System

TSC-147 Joint Tactical Information Distribution System (JTIDS),

TSC-60 
Notes: HF Communications; Manufacturer: Rockwell

TSC-94A SATCOM

TSQ-53 Communications Set

TSQ-111 Technical Control Facility

Notes: Circuit patch and test facilities, test equipment and a 12-line manual switchboard to provide rapid implementation of intra-site and inter-site communication; also equipped with a telephone and teletype order wire facility

TSQ-146(V) Multiplex System

TSQ-165 Modular Air Operations Center (MAOC)

Dimensions: 8 by 24 by 20 feet
Work Stations: 12; Linked by a dual redundant Ethernet Local Area network

Notes: Provides the tactical facilities for the senior element in the Air Force Theater Air Control System. It is the facility that plans, directs and coordinates all aspects of the theater air war. Equipped with state-of-the-art voice communications system providing integrated telephone, radio and intercom functions. A MAOC shelter is an ISO three-to-one expandable, transportable by tactical airlift, sealift, truck or mobilizer.

TTC-30 Telephone Switch

TTC-39A Telephone (Circuit) Switch

TTC-39A(V)1

TTC-39A(V)3

TTC-39A(V)4

TTC-39D

TTC-39E

Notes: A hybrid, mobile, automatic, modular circuit switch that provides both circuit switching service and some nodal control functions for analog and digital traffic.

TTC-41: Telephone System

Notes: Similar to the TTC-39A supporting 160 subscribers

TYC-39 DDN Switching Center

Notes: 50-line, mobile, automatic, electronic store and forward switch under processor control with integral COMSEC and multiplex equipment.

AN/UGC-129(V)1 Information Terminal and Printer

Notes: Terminal and printer used to compose, record, transmit, receive, edit, display, and monitor record message traffic.

UGC-144 Subscriber Terminal

Notes: Three user operated single subscriber terminals (SST) providing a high speed data system capable of composing, editing, storing, displaying, receiving, and transmitting messages into AUTODIN.

URC-119: HF/SSB Radio

Notes: Used for voice communications

URC-119(V2): HF/SSB Radio

Frequency: tunable from 1.6 to 29.99999 KHz (10Hz steps)

Power Output: 500W voice or CW radio system

Notes: controllable from up to one mile with a remote control cable and remote control unit, or telephone lines.

VRC-47 Radio set

Notes: short range, vehicular